# ELE 448 Lab 1

# Due by 05 February, 2020

Instructions:

- Set up Cadence on your account

  - Create a directory in a command window in your home directory, e.g. “mkdir digital_IC”

  - Go to the course website, https://www.ele.uri.edu/courses/ele448/spring.2018/

    - Save this website for future reference

    - Navigate to Lab 1 IC615 Links

- ln -s /net/common/cad/NCSU/DesignKits/ncsu-cdk-1.6.0.beta/cdssetup cdssetup

- ln -s /net/common/cad/NCSU/DesignKits/ncsu-cdk-1.6.0.beta/cdssetup/display.drf display.drf

- ln -s /net/common/cad/NCSU/DesignKits/ncsu-cdk-1.6.0.beta/cdssetup/cdsinit .cdsinit

- ln -s /net/common/cad/NCSU/DesignKits/ncsu-cdk-1.6.0.beta/cdssetup/cdsenv .cdsenv

    - Copy the 4 lines into the cmd window when in the directory that you created in (a)

    - You can now run Cadence by typing IC615 into the linux terminal while in your working directory

  - Save t92y bsim3v3 Parameters to your directory

    - These are the spice parameters that will be used to run simulations in the future so it is useful to have easy access to them

Note:

- Some useful linux instructions are:

  - pwd – present working directory

  - mkdir directory_name – makes a folder, directory_name in the current directory

  - cd /path/to/dir – navigate to the specified directory

  - cd .. – move back to the previous directory

  - ls – lists objects that are in the current directory

- Create a Library in Cadence

  - Go to file  new  Library

  - Add a name for your library and leave the path blank

  - Select attach to existing library

    - Choose TSMC 0.2u to have access to this technology

- Create a schematic for an inverter

  - Select your newly created library

  - Go to file  new  cell view

  - Setup the following

    - Name: Inverter

    - View Name: Schematic

    - Tool: Schematic

  - Create the inverter:

Note:

- Adding parts to your schematic

  - Add  instance (or use the keyboard shortcut, i)

  - Use the component browser to find devices (search NCSU Analog Parts for MOSFETS)

  - Clicking ESC on the keyboard will end the add part instance

- Adding Pins

  - Add  pin (or use the keyboard shortcut, p)

  - Choose the pin name and type

    - Input

    - Output

    - Bidirectional (use for Vdd and Vss pins)

- Other useful shortcuts:

  - w – wire

  - l – label

    - labels can be used to makes connections rather than wires

  - u – undo

  - m – move

  - del – delete

- Create a symbol for your inverter

  - While in the schematic window, go to create  cell view  from cell view

    - The default symbol is a rectangle with the pins you specified in your schematic

    - You can change the symbol as desired to something meaningful

    - Do not label the pins of the symbol to anything other than the way they are labeled in your schematic or else you will encounter errors

- Create a schematic for a 2-input NAND

- Create a symbol for the 2-input NAND

- Create a schematic for a double inverter

- Create a symbol for a double inverter

Lab Write-up:

- Cadence

  - Describe the environment/organization of the software

- Inverter

  - Schematic

    - Provide an image

    - What components were used?

    - Describe how the inverter functions

  - Symbol

    - Provide an image

- Double Inverter

  - Schematic

    - Provide an image

    - What components were used?

    - Describe how this design could be useful

  - Symbol

    - Provide an image

- 2-input NAND

  - Schematic

    - Provide an image

    - What components were used?

    - Describe how the 2-input NAND functions

  - Symbol

    - Provide an image

- Other things to consider in your report

  - Why is the channel width of the PMOS double the width of the NMOS?

Preparation for Next Lab:

- Read sections 2.1 and 2.2 of the lab manual from the course website given above to gain a basic understanding of ring oscillators

  - Note that this lab manual is not up to date, so the actual lab instructions in the manual will differ from this class

  - If time permits, create a schematic and symbol of a ring oscillator that consists of 1 2-input NAND and 10 inverters